Problem Understanding Document

1.Project Title: HT-01 — Remote Post-Operative Recovery Monitor

Team Name: [Project Team — Fight CLub/ HT-01]

Date: 6 April 2026

Tagline: Continuous post-operative oversight at home—vitals, safety, and actionable care—where reliable broadband is not assumed.

2. Introduction

Post-operative monitoring in this project is not limited to a single hospital metric screen. It is framed as extending a “virtual hospital ward” into the patient’s home: the wearable sends heart rate and blood oxygen, supports fall and SOS pathways, and can surface location context for emergencies, while a home gateway relays data to a clinical-style dashboard and enables commands back to hardware (for example, medication dispensing). That design matches the stated motivation in the project materials: high readmission risk when home vitals are missing, and telemedicine that cannot succeed without a concurrent, trustworthy vital-sign stream—especially in rural or semi-urban “grey zones” where continuous connectivity is weak.

The problem matters because the early recovery period combines physiological vulnerability, complex medication schedules, and limited visibility for clinicians once the patient leaves the facility. This project explicitly targets that gap with hardware plus software built for low-bandwidth or intermittent internet scenarios, using long-range radio (LoRa) as the backbone between wearable, gateway, and ancillary nodes.

3. Problem Statement

The solution addresses a structured gap: after discharge, clinical teams lose continuous, objective vital and safety data for patients who live where 4G/5G is unreliable or absent. Standard consumer apps and video visits do not substitute for validated SpO₂, heart rate, and motion-related safety signals in context. At the same time, passive tracking is insufficient for recovery: the system design emphasizes actionability—for example SOS and fall escalation on the gateway and dashboard side, and remote-triggered medication release for adherence-heavy regimens.

Specifically, this implementation targets:

- Absence of continuous home vitals tied to a care workflow (risk of unnoticed deterioration).

- Failure modes of telemedicine when consults are not paired with live physiological data.

- Connectivity asymmetry between urban hospital assumptions and peri-urban/rural home reality, addressed via LoRa mesh-oriented architecture rather than “cloud-only” assumptions.

4. Background & Context

Real-world workflow (typical): A patient is discharged with instructions, medications, and sometimes remote follow-up. In practice, nurses and physicians rely on sporadic phone calls, optional messaging, or clinic visits. Family or ASHA/community health workers may visit, but objective trending of vitals and immediate fall or distress signals are often missing outside the hospital.

Motivation reflected in the project files: The pitch positions the system as universal remote monitoring and telemedicine architecture with chronic disease, post-operative care, and remote consultations as focal areas. The technical folder implements that story concretely: a wrist-style node collects pulse oximetry and motion data and transmits JSON payloads over 433 MHz LoRa; a gateway receives packets, enriches them (for example signal quality, timestamps, local environmental context), and exposes them to a web client via WebSocket; a Python serial bridge can push normalized events into a local-first API backing the Next.js dashboard. Dispenser nodes listen for LoRa JSON commands to actuate servos for specific slots—aligning with the “doctor clicks dispense, pill drops in seconds” narrative. Together, this mirrors a ward-like loop: sense → aggregate → display → command, adapted for home and field conditions.

5. Stakeholders

Patient

- Role: Wears the monitoring device, follows medication plans, may trigger SOS or experience falls during recovery.

- Problems addressed by this design: Limited access to continuous monitoring after surgery; need for credible vital signs during remote follow-up; need for fast escalation (SOS/fall) when they cannot reach a phone; risk of non-adherence to complex post-op medication schedules.

Caregiver (family / home attendant)

- Role: Supports daily recovery, may assist with device charging, observes behavior, coordinates with ASHA or hospital as modeled in the multi-role UI.

- Problems: Uncertainty about objective status (is the patient stable or slowly declining?); stress during emergencies without a clear map/timeline or status channel; difficulty ensuring medication execution for complex regimens—partially mitigated by dispenser integration and dashboard visibility.

Doctor / specialist

- Role: Interprets trends, adjusts care, and in this architecture can drive device-side actions (for example dispense commands relayed through gateway and LoRa).

- Problems: Telemedicine without concurrent vitals; inability to verify adherence or acute change between visits; need for actionable telemetry rather than anecdotal reports—which the system tries to supply through live feeds, risk-oriented UI elements, and command pathways.

Hospital staff (nurses, coordinators, community linkage)

- Role: Triage, discharge planning, linkage to primary health center workflows; in the materials, ASHA visit tracking and paramedic-style handoffs appear as explicit product surfaces.

- Problems: Scaling follow-up across many discharged patients; geographically dispersed populations; need operational views (dashboards, maps, timelines) that match field deployment rather than only in-bed monitoring.

6. Key Challenges Identified

From the system design (not generic):

- Connectivity and store-and-forward reality: The architecture assumes LoRa as a primary transport between edge and gateway; Wi-Fi on the gateway may be intermittent (the firmware supports AP mode for local dashboard attachment). Designing for degraded networks is a core technical challenge.

- Data fusion and integrity: The gateway augments packets with RSSI/SNR, timestamps, and local sensor context; the bridge maps node identifiers to patient records. Maintaining consistent identity and avoiding duplicate or stale vitals in the local database is non-trivial.

- Safety event handling: Fall detection uses a multi-stage motion pipeline (free-fall, impact, inactivity); false positives and false negatives are inherent healthcare-engineering challenges. SOS and fall must surface on the gateway (audio/visual) and downstream to records.

- Bidirectional command safety: Remote dispense is powerful; it requires correct device targeting, auditability, and operational discipline—the command queue pattern in the bridge implies pending → relayed state management.

- Demonstration vs. live clinical use: The gateway includes simulation modes and registry rotation for expos; separating demo behavior from production clinical governance is an explicit lifecycle challenge.

- Regulatory and clinical scope: The stack is engineering-heavy; clinical validation, labeling, and institutional protocols are not replaced by the software alone.

7. Gaps in Existing Systems

Relative to what this project is built to do, common gaps include:

- Cloud-first mobile apps that assume always-on internet and do not integrate LoRa or mesh fallbacks for last-mile homes.

- Passive wearables that stream to a phone but do not tie into a gateway + care dashboard + command loop suitable for facility-style oversight.

- Telemedicine platforms centered on video without synchronized, structured vitals and risk visualization in the same workflow.

- Medication reminders without physical adherence verification at the device—this project’s servo-based dispenser and LoRa command path target that gap for complex regimes.

- Emergency workflows that lack village-scale mapping and multi-role coordination (family, paramedic, ASHA)—the frontend’s routes and components anticipate that operational complexity.

8. Impact of the Problem

If unaddressed, the situation this project targets can lead to:

- Delayed recognition of respiratory compromise, hemodynamic stress, or silent hypoxia after surgery.

- Higher readmission and complication rates when deterioration is noticed late.

- Ineffective remote consultations where neither party has objective data.

- Poor medication adherence in polypharmacy post-op scenarios, affecting pain control, infection prophylaxis, or chronic comorbidity management.

- Geographic inequity: patients in low-connectivity areas receive inferior remote care not because of clinical intent but because infrastructure assumptions fail.

9. Need for the Proposed Solution

The folder’s approach is necessary because it combines:

- Edge sensing aligned with recovery priorities (SpO₂, HR, motion, SOS, with GPS-related hardware in the wrist firmware).

- A gateway that bridges worlds: LoRa in, WebSocket out, optional home Wi-Fi, local alarming, and serial integration with a desktop bridge for EHR-like local persistence.

- A care-facing application with multi-role workflows, maps, risk/trend views, and device/dispenser management—not a single-purpose gadget app.

- Two-way operation: vitals and alerts up, validated commands down—matching the documented “actionable” differentiator.

Without this integrated stack, each layer exists in isolation: sensors without command, dashboards without transport, or connectivity without rural viability.

10. Scope of the System

In scope (as evidenced by implementation and docs):

- Wearable / patient node: Pulse oximetry stack, MPU-based fall logic, SOS/cancel-style inputs, LoRa transmission of JSON vitals, GPS serial path on the wrist firmware.

- Gateway: ESP32-based LoRa receiver, DHT22 environmental sensing, OLED UI with vitals, gateway health, LoRa stats, alert log, simulation page, WebSocket broadcast to clients, Wi-Fi STA + soft AP, serial command channel for LoRa relay and simulation control.

- Dispenser node: ESP8266, four servo channels, LoRa JSON matching device ID and dispense slot.

- Integration layer: Python serial bridge parsing [DATA] lines, posting to /api/local-db, polling pending commands and emitting [CMD]{json} to hardware.

- Web application: Next.js app with routes for dashboard, vitals, patients, emergency, dispensers, devices, LoRa mesh, ASHA visits, family dashboard, paramedic, etc.; local JSON persistence as a Supabase stand-in; risk and trend components, maps, AI insight panels (as described in the project overview).

Out of scope or intentionally limited (inferred from repo):

- Full regulatory clearance, HIPAA-equivalent hosting, and production cloud tenancy—the stack is local-first for development and demo.

- SIM-based cellular gateway (explicitly removed from gateway wiring; software SIM messaging is referenced as simulation in docs).

- End-to-end clinical trials and population outcome studies—not present as artifacts.

- Single unified national EHR—integration is local API and file-backed storage, not a named hospital system.

11. Assumptions

- Devices: ESP32-class gateway, ESP32 wrist band, ESP8266 dispenser; 433 MHz LoRa; sensors as wired in project documentation (MAX30100-family pulse oximetry, MPU6050, DHT22 on gateway, etc.).

- Connectivity: LoRa is available between wearable/dispenser and gateway; Wi-Fi may exist for NTP and upstream use, but AP mode supports local dashboard attachment when needed.

- Users: Operators can run the Python bridge on a PC with USB serial to the gateway while the Next.js app runs locally (referenced API base http://localhost:3000).

- Identity mapping: LoRa node identifiers align with patient identifiers in the local database for meaningful charts and alerts (enforced operationally, not magically guaranteed by hardware alone).

- HT-01 naming: The repository brands the platform “AyuLink”; HT-01 is treated here as the initiative/product identifier for the post-operative recovery monitoring instantiation of that platform.

12. Conclusion

Post-operative recovery is high-stakes and unevenly served when patients leave well-instrumented wards for homes with poor connectivity and limited objective monitoring. This project’s materials and code converge on a coherent response: carry ward-grade signaling (vitals, falls, distress) over LoRa, concentrate it at a gateway that can alarm locally and feed a clinical-style dashboard, and close the loop with remote commands to support adherence—notably via the dispenser node. The problem is important because it ties clinical risk directly to infrastructure reality; the system’s relevance lies in refusing to treat “remote” as synonymous with “smartphone and unlimited data” and instead engineering an end-to-end path from sensor to decision surface to action for post-operative patients in resource-constrained settings.